## LIMITED 1 YEAR WARRANTY

We make every effort to assure that this product meets high quality and durability standards, and warrants to the original purchaser that this product is free from defects in materials and workmanship for the period of one year from the date of purchase (90 days if used as rental equipment). This warranty does not apply to damage due directly or indirectly, to misuse, abuse, negligence or accidents, repairs or alterations outside our facilities, normal wear and tear, or to lack of maintenance. We shall in no event be liable for death, injuries to persons or property, or for incidental, contingent, special or consequential damages arising from the use of our product. Some states do not allow the exclusion or limitation of incidental or consequential damages, so the above limitation of exclusion may not apply to you. THIS WARRANTY IS EXPRESSLY IN LIEU OF ALL OTHER WARRANTIES, EXPRESS OR IMPLIED, INCLUDING THE WARRANTIES OF MERCHANTABILITY AND FITNESS.

To take advantage of this warranty, the product or part must be returned to us with transportation charges prepaid. Proof of purchase date and an explanation of the complaint must accompany the merchandise. If our inspection verifies the defect, we will either repair or replace the product at our election or we may elect to refund the purchase price if we cannot readily and quickly provide you with a replacement. We will return repaired products at our expense, but if we determine there is no defect, or that the defect resulted from causes not within the scope of our warranty, then you must bear the cost of returning the product.

The warranty gives you specific legal rights and you may also have other rights which vary from state to state.

Instruction Manual

Battery Crimping Tool 902-542

Due to continuing improvements, actual product may differ

slightly from the product described herein.

Read this material before using this product.

Failure to do so can result in serious injury.

SAVE THIS MANUAL

TROUBLESHOOTING

| FAULT | ANALYSIS | SOLUTION |
| --- | --- | --- |
| - Tool is inoperative | - Dirt, contaminants, etc in ram area of tool. - The battery is dead - Tool components worn or damaged. | - Clean hydraulic system - Charge the battery - Return tool to manufacturer or distributor |
| - No power during operation or not up to the rated power | - No oil - Dirt, contaminants inside of oil bag - Inner leakage | - Fill oil - Return tool to manufacturer or distributor - Return tool to manufacturer or distributor |
| - Crimp not tight | - Not up to rated power - Error selecting the correct crimping die | - Refer the B - Return tool to manufacturer or distributor |
| - Makes bad crimps (scrap) | - Error selecting the correct crimping die | - Change to correct crimping die |
| - Leakage on the crimping head plunger | - Worn on the sealing kits | - Return tool to manufacturer or distributor |
| - Running of the motor but no power output | - No oil in the oil bag - Air into the hydraulic system - Cold oil | - Fill or change the oil - Return tool to manufacturer or distributor - Use the tool between -20℃-40℃ |

## MAINTENANCE AND SERVICING

Damaged equipment may cause serious personal injury. Do not use damaged equipment. If abnormal noise or vibration occurs, have the problem corrected before further using.

- Before each using, inspect the general condition of the tool. Check for loose screws, misalignment or binding of moving parts, cracked or broken parts, or any other condition that may affect its safe operation.

- After using, clean external surfaces of the tool with a clean, moist cloth, rub rust preventing oil on the metal surface of the tool and the dies to avoid rusting. Store the tool in the dry environment.

- Service to the tool should only be done by a qualified Service Technician.

- In order to prolong the life of the tool please change the oil once per year. Make sure the oil is filtered by 120 mesh net or over 30µm strainer. Meantime avoid getting dust into the oil cup.

- After a long time of use, the sealing kits will be damaged, if there is leakage please contact with the manufacturer and/or the distributor to have the sealing kits changed.

- If the tool is not used for a long time, please make sure the piston stays at its starting position, clean the tool and rub rustproof oil both to the tool piston and accessories. Take out the battery and put them into box and store the tool in a dry surrounding. As following:

## SAVE THIS MANUAL

Keep this manual for the safety warnings and precautions, assembly, operating, inspection,

maintenance and cleaning procedures. Write the month and year of purchase. Keep this

manual and the receipt in a safe and dry place for future reference.

## IMPORTANT SAFETY INFORMATION

In this manual, on the labeling, and all other information

provided with this product:

This is the safety alert symbol. It is used to alert you to potential

personal injury hazards. Obey all safety messages that follow this

symbol to avoid possible injury or death.

DANGER indicates a hazardous situation which,

if not avoided, will result in death or serious injury.

WARNING indicates a hazardous situation which,

if not avoided, could result in death or serious injury.

CAUTION, used with the safety alert symbol,

indicates a hazardous situation which, if not avoided,

could result in minor or moderate injury.

## GENERAL SAFETY RULES

TO WORK IN SAFE CONDITIONS WITH THIS TOOL, IT IS IMPERATIVE TO READ CAREFULLY THE DIRECTIONS FOR USE AND TO FOLLOW THE INSTRUCTIONS IT CONTAINS. IF YOU DO NOT RESPECT THE INFORMATION WRITTEN IN THAT INSTRUCTION MANUAL THE WARRANTY WILL BE CANCELLED.

## 1.  Work area safety

  - Keep work area clean and clear. Cluttered or dark areas invite accidents.

  - This tool is not insulated; please do not use it on live conductor.

  - Please do not use or store the tool under high temperature, or one surrounded by corrosive fluid. Pay attention to the sealing kits becoming old and leaking.

  - Keep children and bystanders away while operating the Battery powered crimping tool. Distractions will cause you to lose control.

  - Do not use or charge the tool around flammable, explosive environment

## 2.  Electrical safety

  - Make sure the plug matches the socket it is used in.  Never try any changes on the plug.

  - Do not put tool, battery and charger in rainy or humid surroundings, it is easy to trigger an electric shock accident if any water goes into the electric system of the tool.

  - Do not use electric wire to carry, pull, or to draw out the plug and do not connect the “-”and “+”, the damaged or twined wire may cause an electric shock accident.

  - If the charger was strongly impacted, or dropped or any other damages happen, please do not try to repair it by yourself, send it back to the authorized service center as soon as possible. A damaged charger may cause an electric shock accident.

Non-interchangeable crimping die, has the feature of automatic centering, four point indent crimping type.

Before crimping

After crimping

Do not slip your finger into the crimping head, due to it is easy to be injured.

During crimping, please make sure the connector with cable is in the center of crimping head, otherwise it will damage the tool or fail                   to crimp.

See the following illustration:

  - The best temperature for charging is between 10℃-40℃. Make sure the air hole of the battery and charger are uncovered during charging.

  - Please cut off the power to the charger each time to reduce the hazard from child or the person who is not expert with the tool.

  - Do not wait until the battery is overcharged, the overcharging may cause the battery to not work any longer. Remove battery from charger as soon as it is fully charged.  Please keep the battery out of the tool to avoid the power discharge.

  - Please do not burn the battery or make it be short-circuited, it may cause an explosion.

  - Do not use the wasted battery otherwise it may cause an electric shock.

  - Do not disassemble the battery and charger. If any problem please contact with manufacturer or distributor for help.

## 3.  Personal safety

  - Stay alert, watch what you are doing and use common sense when operating the tool. Do not use the tool while you are tired or still under the influence of drugs, alcohol or medication. A moment of inattention may result in serious personal injury.

  - Use safety equipment. Always use safety equipment such as mask, helmet, safety cap, insulating shoes and etc to reduce the risk of personal injury.

  - Dress properly. Do not wear loose clothing or jewelry. Keep your hair, clothing and gloves away from moving parts. Loose clothes jewelry or long hair can be caught in moving parts.

  - Maintain power tools. Check for misalignment or binding of moving parts, breakage of parts and any other condition that may affect the tool operation. If damaged, have the tool repaired before use. Many accidents are caused by poorly maintained power tools.

  - Please use the tool properly, the tool with correct power will do the job better and safer at the rate for which it was designed.

  - Do not put your fingers into the head of the tool during operating. Your fingers could be pinched very severely.

## 4.  Service

Have your Battery Crimping Tool serviced by a qualified service technician using only identical replacement parts. This will ensure that the safety of the Battery Crimping Tool Kit is maintained.

## SPECIFIC SAFETY RULES

- Maintain labels and nameplates on the tool which carry important safety information. If unreadable or missing, contact the distributor to replace.

- This product is not a toy, keep it out of reach of children.

- Do not put your fingers into the head of the tool during operating. Your fingers could be pinched very severely.

- Make sure the head was locked firmly during operation.

- Do not use the tool without dies, unless it is the dieless version.

- Do not crimp on live cables or conductors

- Do only crimp copper or Aluminum conducting material

- Do not knock any parts of the tool, otherwise it will cause injury.

- Do not operate at will, it may damage the MCU and electric system

- The design of the limit screw on the head is for preventing the head from dropping or popping.

- Make sure the head was locked firmly during operation.

- Do not use this tool for continuous use. After 30 to 40 cycles, allow the tool to cool for 15 minutes.

- Do not secure this tool in a vise. This tool is designed for hand-held operation.

- The built-in safety valve goes through strict pressure test before marketing; please do not adjust the pressure. If the pressure is not enough please return the tool back to the service center. The tool only can be reused after checking and testing by trained person.

- The warnings, precautions, and instructions discussed in this instruction manual cannot cover all possible conditions and situations that may occur. It must be understood by the operator that common sense and caution are factors which cannot be built into this product, but must be supplied by the operator.

Make sure that all the parts are clean without any oil, dust, especially on the handle and control parts, to avoid dropping the tool during operation.

- First press the  for 3 s to turn on the power, the tool will perform a self inspection for 1 s. the data will show the last record and the tool is ready for working.

inspect if the battery is charged or run out.

- Select the right dies for the intended application,

Don’t operate the tool without dies, unless it is the dieless version.

3) The connecting material shall be positioned in the center crimping head correctly, in order to start the crimping procedure,

4)  A crimping process is initiated by switching the trigger. It is defined by the closing motion of the dies. The connection material is positioned in the stationary half of the crimping dies and the moving part is approaching the compression point.

5)  A crimping cycle is terminated when the dies contacted each other and when the maximum crimping force is reached. After the crimping cycle is completed the piston retracts automatically. Afterwards a new crimping cycle can be initiated or the crimping process can be terminated by opening the latch and remove the connecting material out of the head.

The crimping process can be interrupted at any moment by half releasing the trigger, and the piston will pause for that moment.

Do not put your fingers into the head of the tool during operating. Your fingers could be pinched very severely.

## UNDERSTAND YOUR TOOL

902-542 is a tool for crimping Cu lugs with cables from AWG8 to 450 MCM. It is powered by Li-ion, actuated by motor and controlled by MCU. With a high pressure hydraulic system, LED light, LCD , error warning alarm, pressure display..etc function, it is a perfect tool to be used in electrical construction site.

## Specification

| Max. crimping force: | 6 Tons |
| --- | --- |
| Crimping range: | CU AWG 8-450 MCM |
| Stroke: | 1.38 Inches |
| Crimp/Charge | 250 times(Cu450MCM) |
| Time/crimp: | 3-6s( depending on the size of cable) |
| Hydraulic oil: | Shell Tellus T15# |
| Capacity: | 115ml |
| Ambient temperature | -10-40℃ |
| Voltage: | DC 18V,3Ah |
| Charging voltage: | AC 100V～240V ;50～60Hz |
| Charging time: | Approx.2 hours |
| Accessories: |  |
| Battery: | 2 pcs |
| Charger: | 1 pc |

## Description of components:

| Parts No. | Description | Function |
| --- | --- | --- |
| 1-2 | Die | For crimping, interchangeable die |
| 3 | LCD | Showing the data (crimping times, electricity capacity, error information) |
| 4 | Keyboard | For power switch, function switch, LED light switch |
| 5 | LED indicator | For lighting |
| 6 | Trigger | For operating or retracting the piston in case of incorrect operation. |
| 7 | Lock | Locking the battery |
| 8 | Battery | For supplying power, rechargeable Li-ion（18V）. |

| Set the output force | Press for 3 s to enter setting status, the data will display and be flashing. Press the per time, output data increase by 10 KN (or 5 KN, according to require), this action rotates from Min KN to Max KN. Press for 3 s or leave it for 15 s, and it will return to its initial state. |
| --- | --- |
| LED set | Convenient for working in dark environments, with a light symbol showing on the LCD, long press button to get it to light display. LCD background lighting, meanwhile press the power button, the front LED lights, after 20 s, the light turns off automatically. While the LED is lit, keep pressing to turn off the LED even for the LCD background light. |
| Battery power | full power. , flashing and buzzing means it is time to charge the battery. |
| Error showing | Error happened, such as motor damaged, drive system error and hydraulic system problem… the tool stopped working. LCD showing “error data” as following: it stands for “over power capacity” |
| Power off | Press for 3 s or leave the tool 30 mins to shut off the power automatically. |
| USB port with computer | Remove the battery away from the tool, connect the USB with a computer to exchange the data. LCD shows symbol |

The battery can be used for hundreds of times, when the life span is reduced, change to a new battery.

Please charge the battery in time to avoid it to be used up completely; otherwise it will become useless forever. If the battery is not be used for a long time, it will discharge automatically, so take the battery out of the tool and make sure to charge it one time per quarter.

## Usage of the tool

- LCD board instruction:

- LCD board showing and setting:

Please note: tool in power off , there is no any data showing.

| Power on | Press for 3 s, to get the tool powered up, the keyboard shows all the data for 1 s, then the tool completes its self inspection for 1 s. The data will show last record, the tool is ready to work. |
| --- | --- |
| Change the data | In the power on status, press , the change the data. |

- Function description

MCU –automatically detect the pressure during operation and provide security protection,  shut off the motor and reset automatically after 
            operation.

Auto reset- Release the pressure automatically, retract the piston to the

starting position when reaching the maximum output.

One key control- press the trigger to start working, half pressing the trigger means stop forcing pressure, letting go fully means piston returns to its

original position.

The unit is equipped with a double piston pump which is characterized

by a rapid approach of the die towards the connector and a slow crimping

motion

The head can be smoothly turned by 350° around the longitudinal axis

in order to gain better access to tight corners and other difficult working

areas.

Suspend—finish working or withdraw the hand from the trigger if any accident happens. The device inside of the tool can stop power supply.

One significant sound will be heard and a red display flashes if any error occurs or lower power occurs.( Ref. table 1)

Per each time pressing on the trigger, the LED will light for 20        
seconds.

High-performances Li-ion battery is a strong power supply, No discharge    memory effect., Ultra lower power consumption standby design keeps the

charge for 6 months in storage, meanwhile over discharge device prolongs the life span of the battery.

A temperature sensor makes the tool stop working automatically when

the temperature is over 60℃ for a long time of working, the fault signal

sounds, it means the tool can’t continue work until the temperature reduces to normal.

Micro computer control system with the feature of self-pressure detecting, double protection during operation, counting and realizing the exchange

between computer with data by the converter.

Table 1.

| Series No | Error code |  | Instruction | What it means |
| --- | --- | --- | --- | --- |
| 1 | On --- |  | Self-checking | Self-checking to assure everything is ok |
| 2 | E01 |  | Overload | Hydraulic system may damaged and need check immediately |
| 3 |  | ●●● | Charging signal | Lacking power and needs charging. |
| 4 |  | ●—5sec | Power lacking warning | No power and needs charging immediately |
| 5 | E02 |  | Sensor error | Temperature sensor and power inspecting error |
| 6 | E03 | ●●●● | Temperature warning | Temperature too high, tool needs to cool off |
| 7 | E04 | ●●●●● | No pressure | Motor working but without pressure |

## OPERATING INSTRUCTION

Read the entire SAFETY INFORMATION section at the beginning of this manual including all text under subheadings therein before using this product.

Use this tool for the manufacturer’s intended purpose only. Use other than that which is described in this manual can result in injury or property damage.

1.  Charging

Push battery into charger and connect the plug to the outlet Make sure the room temperature is between 10℃-40℃。The charging time is around 2 hours. Please see the illustration below.